# System Test Case 4 — Network Redundancy and TCP Auto-Recovery

### Test Case ID: STC-004

### Test Title: Network Failover and TCP Persistence on Redundant Ethernet Test

### Test Level: System Test

### Test Type:  Failover / Resilience Testing

### Objective:

To verify that the system automatically switches to the backup Ethernet line without dropping active TCP connections or losing data when the primary link fails.

## Preconditions

- Server has two physical network interfaces (eth0, eth1) configured in "Bonding Mode" (Active-Backup or LACP).

- A continuous TCP data stream (simulating 20 devices) is running.

- Network monitoring tool (Wireshark/Tcpdump) is active.

## Test Inputs

- Continuous 500 TPS flow of stream.

- Physical disconnection or disabling the network interface via software when failure occurs.

Test Steps

| Step No. | Action | Expected Result |
| --- | --- | --- |
| 1 | Verify both network interfaces are UP and traffic is flowing through the Primary (eth0). | Status: UP. Throughput stable. Zero packet loss. |
| 2 | Physically or softwarely (ifconfig eth0 down) disconnect the cable from eth0 OR run | System detects link failure immediately (OS level logs: "Bonding: link status definitely down"). |
| 3 | Observe the data stream continuity. | The TCP connection must not drop. Traffic automatically reroutes to eth1. |
| 4 | Check for data loss or latency spike during the switch. | Handover time < 1 second. TCP retransmission mechanisms handle any in-flight packets. No data loss. |
| 5 | Reconnect eth0 (Restore the primary link). | System detects the link is back. Depending on config, it may switch back to eth0 or stay on eth1. No interruption. |

## Pass/Fail Criteria

- PASS: If the stream continues flowing without breaking the TCP socket connection during the switch.

- FAIL: If the application crashes, the socket disconnects (Connection Reset), or significant data (>1 sec worth) is lost.